FOGPASS VERSION 1.3

In this version I have worked on problem to reduce the gap between the two signals and also I have worked on the volume problem

- gap between the two or more signal for this I have changed the code

in timers.c file

void TIM3_IRQHandler(void)

at line number 215 there is a if condition like

if (k_a >= fileSize)

I have modified to if (k_a >= (fileSize-1600)) I have put this condition because there is a gap between signal

For example if signal audio length is 2 seconds in that two seconds one second it is saying the signal and remaining 1 second it is wasted so to reduce that one second I have put this condition ok

And also in Fat16.c also I am doing that to take less audio compared to original audio

At line number 1306

if (k_a >= (fileSize-1600)) I have added this if condition 1600

in fsmc_nand.c

FSMC_NAND_ReadSmallPage();

At line number 247 I have reduced the delay

this	User_Delay (0xFF);

To

User_Delay (0xF);//JAGADEESH

It is no use but I have kept less delay

in main.c

while();

I have commented out 319 to 367 which is related to warning two because there is no use of this I think so I have commented out this one I have done because from the code given by sir in that there is no use of this warning 2 so I have done this waste of checking condition ok

in route_info.c

Get3LandMarks();

I have commented out form line number 617 to 772 because this is also related to warning 2 so I commented out this one

- audio problem

- in eventlog.h line number 13

this is causing because of there are log events starting from address 0X2000 to  0X5F000

and the volume address is at 0X4000

and there if log events are start writing from 0x2000 address and it also using the 0x4000 and writing 32 every time

so for that I have done   0X5F000 to  0X5D000 and  I have done

volume address  0X4000 to  0X5E000 in menu.h at line number 29 you can observe ok